CDOC216

CHARISSA DATA ACQUISITION SYSTEM

Register Definitions for the LeCroy 4208 TDC

Edition 1.0

April 1995

Nuclear Physics Support Group

Central Laboratory of the Research Councils

Daresbury Laboratory

Registers for the LeCroy 4208 TDC module

This document should be read in conjunction with the document EDOC210 which describes the Register Server implementation for CAMAC.

This is a 8 channel TDC module with CAMAC data readout.

All register attributes include -c (crate address) and -n (station address)

Registers using the xCAMAC class of the CAMAC Register Server

ADC data 			read by f(0) a(x)  or  f(2) a(x)

Addressed readout of the 8 ADC values via the CAMAC dataway ( 2’s complement convention). Channel x (0-7) is selected using subaddress x. Q=0 is returned if an empty channel is read otherwise Q=1 is returned.

Readout using f(2) clears the module at S2.

lc4208#*.adc*.data		attributes  -a x  -f 0  -w 16  -p ro  -l 0  -b 0  -z x

lc4208#*.adc*.data+clr	attributes  -a x  -f 2  -w 16  -p ro  -l 0  -b 0  -z x

Registers using the cCAMAC class of the CAMAC Register Server

These registers do not require data and have no initial value. However read-register operations may return Q and X as data.

reset				write or read by f(9) a(0)

module reset

lc4208#*.reset		attributes  -a 0  -f 9

test LAM			read by f(8) a(0)

returns Q=1 if the module LAM is set

lc4208#*.testlam		attributes  -a 0  -f 8

test LAM and clear	read by f(10) a(0)

returns Q=1 if the module LAM is set and clears the LAM

lc4208#*.testclearlam	attributes  -a 0  -f 10