DOC229

MIDAS

Register Definitions for the LeCroy 4413 module

Edition 1.0

Jan 1996

Nuclear Physics Support Group

Central Laboratory of the Research Councils

Daresbury Laboratory

Registers for the LeCroy 4413 module

This document should be read in conjunction with the document EDOC210 which describes the Register Server implementation for CAMAC.

This is an Updating Programmable Discriminator Module.

All register attributes include -c (crate address) and -n (station address)

Registers using the xCAMAC class of the CAMAC Register Server

Mask Register		read by f(0) a(0) & write by f(16) a(0)

Read the mask register.

lc4413#*.mask		attributes  -a 0  -f 0  -w 16  -p rw  -l 0  -b 0

Threshold Register	read by f(1) a(0) & write by f(17) a(0)

Reads the threshold setting when in local or remote mode. Writes the threshold setting register. 10 bits of data providing 1mV resolution are used to program the common threshold.

lc4413#*.threshold		attributes  -a 1  -f 0  -w 16  -p rw  -l 10  -b 0

Manual Threshold Mode	read by f(1) a(0) & write by f(17) a(0)

When set to 1 the common threshold is set manually (via a front-panel screwdriver control).

lc4413#*.manualmode	attributes  -a 1  -f 0  -w 16  -p rw  -l 1  -b 10  -z d

Registers using the cCAMAC class of the CAMAC Register Server

These registers do not require data and have no initial value. However read-register operations may return Q and X as data.

test				write or read by f(25) a(0)

module test function

lc4413#*.test			attributes  -a 0  -f 25

set local mode		read by f(24) a(0)

lc4413#*.localmode		attributes  -a 0  -f 24

set remote mode	read by f(26) a(0)

lc4413#*.remotemode	attributes  -a 0  -f 26