DOC230

MIDAS

Definitions for the LeCroy 4415A module

Edition 1.0

Jan 1996

Nuclear Physics Support Group

Central Laboratory of the Research Councils

Daresbury Laboratory

Registers for the LeCroy 4415A module

This document should be read in conjunction with the document EDOC210 which describes the Register Server implementation for CAMAC.

This is an Non-Updating Discriminator Module.

All register attributes include -c (crate address) and -n (station address)

Registers using the xCAMAC class of the CAMAC Register Server

Mask Register		write by f(16) a(0)

Write the mask register pattern.

lc4415a#*.mask		attributes  -a 0  -f 16  -w 16  -p wo  -l 0  -b 0

Test Mode			write by f(17) a(0)

Enables Test Mode if =1 and disables Test Mode if = 0.

lc4415a#*.testmode		attributes  -a 0  -f 17  -w 16  -p wo  -l 1  -b 0

Registers using the cCAMAC class of the CAMAC Register Server

These registers do not require data and have no initial value. However read-register operations may return Q and X as data.

test				write or read by f(25) a(0)

module test function

lc4415a#*.test			attributes  -a 0  -f 25

remote/local switch test	read by f(27) a(0)

Returns Q=1 if switch is in REMOTE position otherwise returns Q=0.

lc4415a#*.switchtest		attributes  -a 0  -f 27